Sheetpress system experience

We build the Sheetpress system with a couple of upgrades stated below. After using it for a little over a month we are still really pleased with the system. The simplicity of the design is great and works very fine. The learning curve to get good results is quite o.k.  See below an overview of the updates we did om our design and suggestions for further future improvements.

Updates V4.1 compared to v4.0

- Wheels added for transportability

- KW electricity meter added with resettable day-counter. This means you can measure how KW of electricity you have after a day of production.

- All sheets and tubes are modelled to be suitable for lasercutting and sheet-bending

- Changing hydraulic piston for a model witch can be operated by compressed air for ease of use.

- Added end-stops for sheetpress and coolpress so plates can eb slided through more easily

- Updated Bom-list with specific suppliers

- Electronics console with de-attachable electronics box

- Adding bars to control the release and pressure building of the hydraulic piston.

Future improvements suggestions

- When heating up we encounter that a good protocol is necessary. When setting both plates simultaneously to the desired temperature the top plate overheats enormously about 60-70 degrees above the desired temperature. This could result in overheating the machine and permanently damaging it. There should be a system which prevents this from happening or a sensor which can shut down the system when the overheating happens.

- The Cold-press should also have a spring similar to the Hot-press on the hydraulic press to prevent it from overloading and providing too much stress on the construction

What went wrong during the build

- Poor documentation about assembling the heat cartridges in the 25mm aluminium blocks. Should be more clear how precise the drilling and reaming of the blocks should be in order to assemble the heating elements. Our solution after a lot of trial and error was reaming the hole to 11.95mm. Then heating the blocks to 250 degrees (this temporarily expands the hole by 0.01/0.02mm), while hot pressure fit the heating elements in the block.

- Several parts missed on the BOM-list regarding the electric components these are added in attached list.

- Electric schematics had some mistakes and didn’t work as proposed see attached an updated version.

Melting experience

- We tried several types of plates for sandwiching the plastics for melting, Aluminum, Steel & RVS in various thicknesses and several finishes from polished, standard to sanded. Till now we discovered the best and most convenient material is aluminium since the weight is much lower than steel. We recommend to sand it with a rotex 120 after several melting cycles to make sure the mould release will keep having grip.

Price calculation

We took track of all our hours which we put into this project. See below an excel which you can use. By choosing a specific hour rate you can calculate what the total price will be for building this machine.